# Multi-UAV Path Planning — Complete Step-by-Step Guide

### From Zero to Working Demo

Author: Awais Shah
Project: Multi-UAV Path Planning for Urban Air Mobility — Phase 1
Date: July 2026

## Table of Contents

- Choose Your MATLAB Option

- Create a MathWorks Account

- Option A: MATLAB Online Setup

- Option B: MATLAB Desktop Setup

- Install Required Toolboxes

- Verify Toolbox Installation

- Upload / Place Project Files

- Understand the Project Files

- Run the Simulation

- What You Should See

- Troubleshooting Common Errors

- Record Your Demo Video

- Next Steps (Phase 2+)

## Step 1 — Choose Your MATLAB Option

You have two ways to use MATLAB. Pick one:

| Option | Cost | Pros | Cons |
| --- | --- | --- | --- |
| MATLAB Online (browser) | Free tier: 20 hrs/month | No install needed, works on any PC | Limited hours, slower for heavy computation |
| MATLAB Desktop (installed) | Free 30-day trial, or student license (~$50/year) | Unlimited hours, faster | Large download (~10-20 GB), needs decent PC |

Recommendation for this project: Start with MATLAB Online to avoid wasting time on installation. Switch to Desktop if you run out of hours.

## Step 2 — Create a MathWorks Account

- Open your browser and go to: https://www.mathworks.com/mwaccount/register

- Fill in:

  - Email: Use your university email if you have one (gives you access to student licenses)

  - Country, Name, Password — fill as normal

- Click Create → check your email inbox → click the verification link

- You now have a MathWorks account

Important: Remember your MathWorks email and password. You’ll need them for everything.

## Step 3A — MATLAB Online Setup

Skip this section if you chose Desktop installation (go to Step 3B).

- Go to: https://matlab.mathworks.com/

- Sign in with your MathWorks account

- Click Open MATLAB Online

- Wait for the browser-based MATLAB environment to load (takes 30-60 seconds)

- You should see:

  - Command Window (bottom) — where you type commands

  - Current Folder panel (left) — your file browser

  - Editor (center/right) — where you open and edit .m files

Note: Your 20 free hours start counting only when MATLAB Online is open. Close the tab when you’re not using it.

Done! Jump to Step 4.

## Step 3B — MATLAB Desktop Installation

Skip this section if you chose MATLAB Online (go to Step 4).

### Download MATLAB

- Go to: https://www.mathworks.com/downloads/

- Sign in with your MathWorks account

- Select MATLAB R2024b (or the latest version shown)

- Click Download for Windows (or Mac/Linux as applicable)

- The installer file will download (~300 MB initial downloader)

### Install MATLAB

- Run the downloaded installer (matlab_R2024b_win64.exe)

- Sign in with your MathWorks account when prompted

- Accept the license agreement

- License selection:

  - If you have a student license → select it

  - If you’re using a trial → select “30-Day Trial”

- Product selection screen — This is critical. Check these boxes:

  - ☑ MATLAB (always included)

  - ☑ UAV Toolbox

  - ☑ Navigation Toolbox

  - ☑ Sensor Fusion and Tracking Toolbox (needed for Phase 2+)

  - ☑ Simulink (useful later, optional for Phase 1)

- Click Install → wait for download and installation (10-20 GB, takes 30-90 minutes depending on internet speed)

- When finished, click Close and launch MATLAB from your Start Menu / Desktop shortcut

Done! Jump to Step 5.

## Step 4 — Install Required Toolboxes

### If Using MATLAB Online:

MATLAB Online includes many toolboxes by default. To check/add:

- In MATLAB Online, click the “Add-Ons” button in the top toolbar (or go to Home tab → Add-Ons → Get Add-Ons)

- Search for each of these and click “Add” if not already installed:

  - UAV Toolbox

  - Navigation Toolbox

  - Sensor Fusion and Tracking Toolbox

- If prompted, agree to any license terms and wait for installation

### If Using MATLAB Desktop:

- In MATLAB, go to Home tab → Add-Ons → Get Add-Ons

- The Add-On Explorer opens

- Search for: UAV Toolbox → Click Install

- Search for: Navigation Toolbox → Click Install

- Search for: Sensor Fusion and Tracking Toolbox → Click Install

- MATLAB may ask you to restart — do it

## Step 5 — Verify Toolbox Installation

This step confirms everything is installed correctly before you run the project.

### Test 1: Check installed toolboxes

Type this in the MATLAB Command Window and press Enter:

ver

You should see a list that includes at least:

MATLAB                                   Version X.X
UAV Toolbox                              Version X.X
Navigation Toolbox                       Version X.X

If any of these are missing, go back to Step 4.

### Test 2: Check that key functions exist

Type each of these one at a time and press Enter:

help uavScenario

→ Should print the help text for uavScenario (not an error)

help occupancyMap3D

→ Should print help text

help plannerRRTStar

→ Should print help text

help stateSpaceSE3

→ Should print help text

### Test 3: Quick smoke test

Type this and press Enter:

scene = uavScenario('UpdateRate', 10);
disp('UAV Toolbox is working!')

If you see UAV Toolbox is working! — you’re all set.
If you see an error like Undefined function 'uavScenario' — the UAV Toolbox is not installed. Repeat Step 4.

## Step 6 — Upload / Place Project Files

Your project has 4 MATLAB script files. They must all be in the same folder.

### File list:

| File | Purpose |
| --- | --- |
| main_phase1.m | The main script you run — configures everything and calls the others |
| create_city_scenario.m | Builds the 3D city with 9 buildings and a ground plane |
| plan_uav_path.m | Plans a collision-free path using the RRT* algorithm |
| animate_uav.m | Animates the drone flying the planned path in 3D |

### If Using MATLAB Online:

- In the Current Folder panel (left side), click the Upload button (↑ icon)

- Select all 4 .m files from your computer:

  - main_phase1.m

  - create_city_scenario.m

  - plan_uav_path.m

  - animate_uav.m

- Click Open → files appear in your MATLAB Online workspace

### If Using MATLAB Desktop:

- Create a project folder, e.g.: C:\Users\YourName\Documents\MATLAB\UAV_PathPlanning\

- Copy all 4 .m files into that folder

- In MATLAB, navigate to that folder:

  - Use the Current Folder panel on the left to browse to it, OR

  - Type in the Command Window:

- cd 'C:\Users\YourName\Documents\MATLAB\UAV_PathPlanning'

- Verify the files are visible in the Current Folder panel

## Step 7 — Understand the Project Files

Before running anything, here’s what each file does and how they connect:

### How the files work together:

main_phase1.m  (you run THIS)
    │
    ├──► create_city_scenario.m
    │        Creates 9 buildings in a 3x3 grid
    │        Creates a 3D occupancy map (marks where buildings are)
    │        Creates a green ground plane
    │        Creates the quadrotor drone object
    │        Returns: scenario, map3D, uav
    │
    ├──► plan_uav_path.m
    │        Takes the 3D occupancy map
    │        Inflates it by 1.5m (safety buffer around buildings)
    │        Uses RRT* algorithm to find a collision-free 3D path
    │        Returns: waypoints (the path), solnInfo (success/fail)
    │
    └──► animate_uav.m
             Takes the scenario, drone, and waypoints
             Calculates speed-based timing for each waypoint
             Opens a 3D figure window
             Plots the planned path in green
             Marks Start (green square) and Goal (red star)
             Animates the drone flying along the path
             Drone faces its direction of travel

### Key concepts explained:

**What is RRT*?**
RRT* (Rapidly-exploring Random Tree Star) is a path planning algorithm. It randomly samples points in 3D space, builds a tree of collision-free connections, and finds the shortest path from start to goal while avoiding obstacles (buildings). The * means it optimizes the path over iterations — more iterations = shorter/smoother path.

What is an occupancy map?
A 3D grid where each cell is marked as “occupied” (inside a building) or “free” (empty air). The RRT* planner uses this map to check if a path segment would crash into a building.

What does “inflate” mean?
When we inflate the occupancy map by 1.5 meters, every building grows 1.5m in all directions on the map. This means the drone will never fly closer than 1.5m to any building surface — a safety buffer.

What is the move() function?
Each frame of the animation, we calculate where the drone should be (by interpolating along the planned path) and tell MATLAB to place the drone there using move(uav, motionVector). The motion vector has 16 numbers: position(3), velocity(3), orientation-quaternion(4), angular-velocity(3), acceleration(3).

## Step 8 — Run the Simulation

### Pre-flight checklist:

- MATLAB is open (Online or Desktop)

- All 4 .m files are in your current folder

- Current Folder panel shows: main_phase1.m, create_city_scenario.m, plan_uav_path.m, animate_uav.m

- Toolboxes are installed (Step 5 passed)

### Run it:

- Type the following in the Command Window and press Enter:

main_phase1

- Wait. The simulation has three stages:

  - Stage 1 (instant): “Initializing city block scenario…” — builds the 3D world

  - Stage 2 (5-30 seconds): “Computing RRT* path…” — the planner searches for a collision-free route. This takes time because RRT* runs 3000 iterations.

  - Stage 3 (30-60 seconds): “Starting UAV flight animation…” — a 3D window opens and the drone flies

- Do not close anything while it’s running. The figure window will appear automatically.

## Step 9 — What You Should See

### Command Window Output:

Initializing city block scenario...
Computing RRT* path...
Planning path from [5.0, 5.0, 10.0] to [90.0, 90.0, 15.0]...
Path found successfully in XXXX iterations!
Animating flight simulation...
Starting UAV flight animation (XX.X s at 3.0 m/s)...
Animation complete.

### 3D Figure Window:

A window opens showing:

- Green ground plane — the floor of the city

- 9 gray cuboid buildings — arranged in a 3×3 grid with streets between them

- Green line with circle markers — the RRT* planned path weaving through the city

- Green square marker — the Start position at [5, 5, 10]

- Red star marker — the Goal position at [90, 90, 15]

- Blue quadrotor drone — flying along the green path, facing its direction of travel

The drone should: - Start near the bottom-left corner - Navigate through the streets between buildings - Possibly fly over shorter buildings if it’s more efficient - Arrive at the top-right corner

Note: Because RRT* is a randomized algorithm, the path will look slightly different every time you run it. This is normal and expected.

### Interacting with the 3D view:

- Rotate: Click and drag inside the figure window

- Zoom: Scroll mouse wheel

- Pan: Hold Shift + click and drag

- Reset view: Click the “Home” button in the figure toolbar

## Step 10 — Troubleshooting Common Errors

### Error: Undefined function or variable 'uavScenario'

Cause: UAV Toolbox is not installed.
Fix: Go back to Step 4 and install the UAV Toolbox.

### Error: Undefined function or variable 'plannerRRTStar'

Cause: Navigation Toolbox is not installed.
Fix: Go back to Step 4 and install the Navigation Toolbox.

### Error: Undefined function or variable 'stateSpaceSE3'

Cause: Navigation Toolbox is not installed or is an older version.
Fix: Update the Navigation Toolbox via Add-Ons → Check for Updates.

### Warning: RRT* failed to find a collision-free path

Cause: The planner couldn’t find a valid route in 3000 iterations. This is rare with the current building layout but can happen.
Fix: Run main_phase1 again (different random seed = different result). If it keeps failing, increase the iteration limit in plan_uav_path.m line 41:

planner.MaxIterations = 5000;   % was 3000, increased

### Error: Index exceeds the number of array elements

Cause: Usually means the path planner returned an empty path.
Fix: Same as above — re-run or increase iterations.

### The 3D figure is blank or shows only the ground

Cause: The camera might be positioned underground or far away.
Fix: In the figure window, click the Rotate 3D button in the toolbar, then click and drag to reorient the view. Or type in Command Window:

view(45, 30);

### MATLAB Online is very slow / animation stutters

Cause: MATLAB Online runs in a browser with limited GPU acceleration.
Fix: - Close other browser tabs to free memory - Reduce map resolution in main_phase1.m line 23: change mapResolution = 1 to mapResolution = 0.5 - Or switch to MATLAB Desktop for smoother animation

### Error: Unable to perform assignment because the left and right sides have a different number of elements

Cause: A function returned an unexpected format.
Fix: Make sure all 4 .m files are in the same folder and you haven’t accidentally modified any of them.

## Step 11 — Record Your Demo Video

Once the simulation runs successfully, record a demo video for your portfolio.

### On Windows:

- Press Win + G to open the Xbox Game Bar

- Click the Record button (or press Win + Alt + R)

- Run main_phase1 in MATLAB

- When the 3D animation finishes, press Win + Alt + R again to stop recording

- The video is saved to: C:\Users\YourName\Videos\Captures\

### On Mac:

- Press Cmd + Shift + 5 to open Screen Recording

- Select the MATLAB window → click Record

- Run main_phase1

- Click the Stop button in the menu bar when done

### Tips for a good demo video:

- Before recording, rotate the 3D view to a nice angle

- Run the simulation once first (without recording) so you know the timing

- Add narration or captions explaining what’s happening:

  - “This is a 3D city environment with 9 buildings”

  - “The RRT* algorithm plans a collision-free path (shown in green)”

  - “The blue quadrotor drone autonomously follows the planned path”

## Step 12 — Next Steps

Phase 1 is complete once you have a working demo. Here’s what comes next:

### Phase 2: Multi-Drone Extension (~5-6 hours)

- Add 2-3 more drones with different start/goal positions

- Plan paths sequentially (drone 1’s path becomes an obstacle for drone 2)

- Visualize all drones flying simultaneously without collisions

### Phase 3: Dynamic Avoidance (~4-5 hours)

- Introduce a surprise obstacle mid-flight

- Implement reactive replanning when an obstacle appears

- This is the “wow moment” for your demo video

### Phase 4: Polish & Packaging (~3-4 hours)

- Better colors, camera angles, path visualization

- Record a polished 1-3 minute demo video

- Write a README and push to GitHub

## Quick Reference Card

| Action | Command |
| --- | --- |
| Run the simulation | main_phase1 |
| Check installed toolboxes | ver |
| Clear everything and start fresh | clear; clc; close all; |
| Change the drone’s start position | Edit uavStart in main_phase1.m line 32 |
| Change the drone’s goal position | Edit uavGoal in main_phase1.m line 33 |
| Change the drone’s speed | Edit uavSpeed in main_phase1.m line 29 |
| Increase planning accuracy | Increase MaxIterations in plan_uav_path.m line 41 |
| Add/remove buildings | Edit the buildingParams section in main_phase1.m lines 39-54 |
| Change safety buffer | Edit safetyDistance in main_phase1.m line 26 |

End of Guide